Radiation Safety Competency Assessment

KAP Questionnaire — Cardiac Catheterization Laboratory Personnel

Version 0.1 — DRAFT FOR EXPERT REVIEW AND CONTENT VALIDATION

Status. Every item in this instrument — including the provisional answer key — must pass expert-panel review (content validity), pilot testing, and reliability/validity assessment before use. Items flagged ⚠ require particular expert attention: answers must be verified against current ICRP publications, OSHA 29 CFR 1910.1096, IAEA GSR Part 3, and Saudi NRRC/SFDA regulatory limits. An Arabic version (forward–backward translation) is planned after content validation.

Structure:  Section A — Professional & demographic (11 items) · Section B — Knowledge (20 items, single best answer + "I don't know") · Section C — Attitude (13 items, 5-point Likert) · Section D — Practice (12 items, 5-point frequency) · Section E — Previous training (9 items).

# Section A — Professional and Demographic Variables

A1.  Professional role:

☐ Interventional cardiologist   ☐ Cardiology fellow/resident   ☐ Electrophysiologist   ☐ Nurse   ☐ Radiographer / radiologic technologist   ☐ Cardiovascular technician   ☐ Anesthesia staff   ☐ Other: ____________

A2.  Primary specialty / department:

________________________________________

A3.  Years of professional experience:

☐ <2   ☐ 2–5   ☐ 6–10   ☐ 11–20   ☐ >20

A4.  Years working in radiation-based practice (cath lab / fluoroscopy environment):

☐ <2   ☐ 2–5   ☐ 6–10   ☐ 11–20   ☐ >20

A5.  On average, how many radiation-based procedures do you participate in per week?

☐ <5   ☐ 5–10   ☐ 11–20   ☐ >20

A6.  Types of procedures you routinely perform or attend (select all that apply):

☐ Diagnostic coronary angiography   ☐ PCI   ☐ Electrophysiology / ablation   ☐ Device implantation (PPM/ICD/CRT)   ☐ Structural interventions (TAVI, ASD/PFO, etc.)   ☐ Peripheral/vascular interventions   ☐ Other: ____________

A7.  Does your institution have a formal radiation-safety program?

☐ Yes   ☐ No   ☐ I don't know

A8.  Is a personal dosimeter (e.g., TLD badge) assigned to you?

☐ Yes   ☐ No   ☐ I don't know

A9.  If yes, how often do you actually wear it?

☐ Always   ☐ Often   ☐ Sometimes   ☐ Rarely   ☐ Never

A10.  Which protective equipment is available in your cath lab? (select all that apply)

☐ Lead aprons   ☐ Thyroid shields   ☐ Lead glasses   ☐ Ceiling-suspended shield   ☐ Table-side lead drapes   ☐ Mobile lead barrier   ☐ Radiation-protective caps   ☐ I don't know

A11.  Sex:  ☐ Male   ☐ Female

Age group:  ☐ <30   ☐ 30–39   ☐ 40–49   ☐ ≥50

# Section B — Knowledge

Instructions: choose the single best answer. If you do not know, please select "I don't know" rather than guessing.

Expert-review copy. Provisional correct answers are shown in bold and marked ✓. Participant copies will contain no answer markings. ⚠ = item whose key must be verified against current standards.

B1. 
      The type of radiation used in fluoroscopy is:

☐  a)  Ultrasound

☐  b)  X-rays (ionizing radiation)  ✓

☐  c)  Magnetic fields

☐  d)  Infrared radiation

☐  I don't know

B2. 
      During fluoroscopy, the main source of scattered radiation to staff in the cath lab is:

☐  a)  The X-ray tube housing

☐  b)  The image detector

☐  c)  The patient's body  ✓

☐  d)  The control room

☐  I don't know

B3. 
      If you double your distance from the radiation source, your radiation exposure becomes approximately:

☐  a)  One half

☐  b)  One quarter  ✓

☐  c)  One eighth

☐  d)  Unchanged

☐  I don't know

B4. 
      The three fundamental principles of radiation protection are:

☐  a)  Speed, accuracy, planning

☐  b)  Time, distance, shielding  ✓

☐  c)  Collimation, magnification, angulation

☐  d)  Dose, rate, energy

☐  I don't know

B5. 
      The unit used to express occupational effective radiation dose is:

☐  a)  Becquerel

☐  b)  Gray

☐  c)  Sievert (mSv)  ✓

☐  d)  Watt

☐  I don't know

B6. ⚠ The ICRP-recommended occupational effective dose limit is:

☐  a)  1 mSv per year

☐  b)  20 mSv per year (averaged over 5 years)  ✓

☐  c)  100 mSv per year

☐  d)  There is no limit

☐  I don't know

B7. ⚠ The current ICRP-recommended annual equivalent dose limit for the lens of the eye (occupational) is:

☐  a)  150 mSv

☐  b)  20 mSv  ✓

☐  c)  500 mSv

☐  d)  1 mSv

☐  I don't know

B8. ⚠ After a pregnancy is declared, the recommended dose limit to the fetus for the remainder of the pregnancy is approximately:

☐  a)  1 mSv  ✓

☐  b)  20 mSv

☐  c)  50 mSv

☐  d)  No specific limit

☐  I don't know

B9. 
      Radiation-induced cancer is classified as which type of radiation effect?

☐  a)  Deterministic (tissue reaction)

☐  b)  Stochastic  ✓

☐  c)  Acute

☐  d)  Mechanical

☐  I don't know

B10. 
      Radiation-induced cataract is best classified as:

☐  a)  A deterministic effect (tissue reaction)  ✓

☐  b)  A stochastic effect with no threshold

☐  c)  Not related to radiation

☐  d)  An infectious complication

☐  I don't know

B11. 
      Scattered radiation exposure to the operator is generally highest when standing:

☐  a)  On the image-detector side of the patient

☐  b)  On the X-ray tube side of the patient  ✓

☐  c)  At the patient's feet

☐  d)  Position makes no difference

☐  I don't know

B12. 
      Using image magnification during fluoroscopy generally:

☐  a)  Decreases patient dose

☐  b)  Increases radiation dose  ✓

☐  c)  Has no effect on dose

☐  d)  Only affects image brightness

☐  I don't know

B13. 
      Steeply angulated projections (e.g., LAO cranial) generally:

☐  a)  Reduce operator exposure

☐  b)  Increase operator and patient exposure  ✓

☐  c)  Have no effect on exposure

☐  d)  Eliminate scatter

☐  I don't know

B14. 
      Collimating the X-ray beam to the area of interest:

☐  a)  Increases scatter

☐  b)  Reduces dose and improves image quality  ✓

☐  c)  Increases patient dose

☐  d)  Is only cosmetic

☐  I don't know

B15. 
      Cine acquisition (recording), compared with standard fluoroscopy, delivers a radiation dose that is:

☐  a)  Lower

☐  b)  About the same

☐  c)  Substantially higher  ✓

☐  d)  Zero

☐  I don't know

B16. ⚠ Which parameter best reflects the total radiation output delivered during a procedure?

☐  a)  Fluoroscopy time alone

☐  b)  Dose–area product (DAP/KAP)  ✓

☐  c)  Number of cine runs alone

☐  d)  Table height

☐  I don't know

B17. ⚠ The typical effective dose to the patient from a diagnostic coronary angiography is on the order of:

☐  a)  0.02 mSv (like a chest X-ray)

☐  b)  ~5–7 mSv (equivalent to several hundred chest X-rays)  ✓

☐  c)  100 mSv

☐  d)  1000 mSv

☐  I don't know

B18. 
      A standard lead apron (0.25–0.5 mm lead equivalent) attenuates approximately what proportion of scattered radiation?

☐  a)  About 10%

☐  b)  About 50%

☐  c)  90% or more  ✓

☐  d)  100% (complete protection)

☐  I don't know

B19. 
      The purpose of the ceiling-suspended lead shield is to:

☐  a)  Protect the patient from the primary beam

☐  b)  Reduce scattered radiation reaching the operator's head, eyes, and upper body  ✓

☐  c)  Improve image quality

☐  d)  Support the image detector

☐  I don't know

B20. ⚠ Wearing leaded glasses primarily protects against:

☐  a)  Skin injury of the hands

☐  b)  Radiation-induced cataract of the eye lens  ✓

☐  c)  Thyroid cancer

☐  d)  Bone marrow suppression

☐  I don't know

# Section C — Attitude

Instructions: rate your agreement with each statement.  1 = Strongly disagree, 2 = Disagree, 3 = Neutral, 4 = Agree, 5 = Strongly agree

| # | Statement | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| C1 | Radiation safety is an essential part of my clinical practice, not a secondary technical issue. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C2 | I am personally at risk of health effects from occupational radiation exposure. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C3 | My colleagues in the cath lab are at risk from occupational radiation exposure. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C4 | Patients may be exposed to unnecessary radiation if safety practices are inadequate. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C5 | Long-term occupational radiation exposure can have serious health consequences. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C6 | Radiation protection is my personal responsibility, not only the institution's. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C7 | Reducing unnecessary radiation exposure is important even when it takes extra time or effort. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C8 | I am motivated to improve my radiation-safety knowledge. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C9 | I am willing to attend a dedicated radiation-safety educational course. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C10 | I have previously made an effort to seek information about radiation risks on my own. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C11 | The radiation-safety education currently provided by my institution is adequate. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C12 | Standardized radiation-safety education should be required for all cath lab personnel. | ☐ | ☐ | ☐ | ☐ | ☐ |
| C13 | Competency-based radiation-safety certification (renewed periodically, similar to BLS) should be required before independent practice in radiation-based environments. | ☐ | ☐ | ☐ | ☐ | ☐ |

# Section D — Practice (Self-Reported)

Instructions: how often do you do each of the following during radiation-based procedures?  1 = Never, 2 = Rarely, 3 = Sometimes, 4 = Often, 5 = Always

| # | Behavior | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| D1 | I wear a lead apron during procedures. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D2 | I wear a thyroid shield during procedures. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D3 | I wear lead glasses during procedures. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D4 | I position the ceiling-suspended shield / available shielding between myself and the radiation source. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D5 | I use table-side lead drapes when available. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D6 | I deliberately position myself to reduce my exposure (e.g., away from the tube side). | ☐ | ☐ | ☐ | ☐ | ☐ |
| D7 | I step back or maximize my distance from the source during cine acquisition when my role allows. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D8 | I remain aware of where scattered radiation is highest during the procedure. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D9 | I actively try to minimize fluoroscopy time (or support the operator in doing so). | ☐ | ☐ | ☐ | ☐ | ☐ |
| D10 | I monitor or check the radiation dose indicators (fluoro time, DAP, air kerma) during or after procedures. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D11 | I wear my personal dosimeter (TLD badge) as required. | ☐ | ☐ | ☐ | ☐ | ☐ |
| D12 | I speak up or take action when I notice unsafe radiation practices in the lab. | ☐ | ☐ | ☐ | ☐ | ☐ |

Objective practice checklist. Where feasible, items D1–D7 and D11 will also be assessed by direct structured observation by the research team. Self-reported and observed practice will be analyzed separately.

# Section E — Previous Radiation-Safety Training

E1.  Have you ever received formal radiation-safety education or training?

☐ Yes   ☐ No

E2.  If yes, what type? (select all that apply)

☐ Dedicated radiation-safety course   ☐ Institutional orientation / in-service training   ☐ Part of professional degree curriculum   ☐ Online course   ☐ Conference / workshop   ☐ Other: ____________

E3.  When was your most recent radiation-safety training?

☐ Within the last year   ☐ 1–3 years ago   ☐ 3–5 years ago   ☐ More than 5 years ago   ☐ Never

E4.  Approximate total duration of your previous radiation-safety training:

☐ None   ☐ <1 day   ☐ 1 day   ☐ 2–5 days   ☐ >5 days

E5.  How often does your institution provide radiation-safety education?

☐ Regularly (at least yearly)   ☐ Occasionally   ☐ Rarely   ☐ Never   ☐ I don't know

E6.  Was radiation safety formally included in your professional training (residency / fellowship / nursing / technologist program)?

☐ Yes   ☐ No   ☐ Partially

E7.  Do you consider your previous radiation-safety education adequate for your current work?

☐ Yes   ☐ No   ☐ Unsure

E8.  Are you interested in receiving additional radiation-safety training?

☐ Yes   ☐ No   ☐ Unsure

E9.  Have you ever been formally assessed (tested/certified) on radiation safety?

☐ Yes   ☐ No

# Standards and Regulatory References

The answer key, competency benchmarks, and educational course content are anchored to the following standards. The expert panel is asked to verify all cited limits against the current editions.

| Reference | Key content used by this instrument |
| --- | --- |
| ICRP Publication 103 (2007) | Occupational effective dose limit 20 mSv/year averaged over 5 years (max 50 mSv in any single year); ALARA / optimization principle (item B6). |
| ICRP Publication 118 | Eye-lens equivalent dose limit 20 mSv/year; cataract as a tissue reaction (items B7, B10, B20). |
| ICRP Publication 120 | Radiological protection in cardiology — cath-lab-specific guidance (items B11–B19). |
| OSHA 29 CFR 1910.1096 — Ionizing Radiation | US occupational standard: quarterly whole-body dose limit of 1.25 rem (12.5 mSv); mandatory personal monitoring when a worker is likely to receive more than 25% of the quarterly limit; radiation-area caution signage; mandatory instruction of all personnel in radiation-safety practices and hazards — a regulatory precedent for this study's competency-based education model. |
| IAEA GSR Part 3 | International Basic Safety Standards — the framework most national regulators, including Saudi Arabia, align with. |
| Saudi NRRC / SFDA regulations | Governing national requirements for the participating centers; panel to confirm current dose limits and monitoring requirements. |

Reconciliation note for the expert panel. OSHA's numerical limits predate current ICRP recommendations and are less restrictive (12.5 mSv/quarter vs. 20 mSv/year). The questionnaire teaches the ICRP/IAEA/NRRC limits as operative, while OSHA 1910.1096 is cited for its monitoring, signage, and mandatory personnel-instruction requirements, which directly support the study's Goal 3 (competency-based certification).

## Administration Notes

–  Estimated completion time: 12–15 minutes (confirm at pilot).

–  Anonymized participant codes link Phase II questionnaires with TLD and procedural data.

–  The same instrument is administered pre- and post-course in Phase II; knowledge item order may be shuffled post-course to limit recall effects (decision pending expert review).

–  Delivery mode (paper vs. electronic) to be decided per center; identical wording across centers is mandatory.